# Q1: Implement NOT, AND, OR Gates Using Only NAND Gates

# 1. Aim

To implement NOT, AND, and OR logic gates using only NAND gates in HDL, draw their schematic diagrams, verify their Boolean expressions, and observe the simulation output using suitable test inputs.

# 2. Schematic Diagram

## A. NOT Gate using NAND

A NAND gate with both inputs connected together behaves as a NOT gate.

A ─────┐
              │
              ▼
           ┌───────┐
       A ─►│ NAND  │────── Y
           └───────┘

Boolean Expression:
Y = (A · A)' = A'

## B. AND Gate using NAND

An AND gate can be made using two NAND gates.

A ───────┐
         ▼
      ┌───────┐
B ───►│ NAND  │──── X ───┐
      └───────┘          ▼
                       ┌───────┐
                 X ───►│ NAND  │──── Y
                       └───────┘
                         ▲
                         │
                         X

Boolean Expression:
X = (A · B)'
Y = (X · X)'
Y = ((A · B)')'
Y = A · B

## C. OR Gate using NAND

An OR gate can be implemented using three NAND gates.

┌───────┐
A ──────────►│ NAND  │──── A'
             └───────┘
                 │
                 │       ┌───────┐
                 └──────►│       │
                         │ NAND  │──── Y
                 ┌──────►│       │
                 │       └───────┘
             ┌───────┐       ▲
B ──────────►│ NAND  │──── B'│
             └───────┘       │

Boolean Expression:
A' = (A · A)'
B' = (B · B)'
Y = (A' · B')'
Y = A + B

# 3. HDL Code

// NOT Gate using NAND
module not_using_nand(
    input A,
    output Y
);
    nand(Y, A, A);
endmodule


// AND Gate using NAND
module and_using_nand(
    input A,
    input B,
    output Y
);
    wire X;

    nand(X, A, B);
    nand(Y, X, X);
endmodule


// OR Gate using NAND
module or_using_nand(
    input A,
    input B,
    output Y
);
    wire A_not, B_not;

    nand(A_not, A, A);
    nand(B_not, B, B);
    nand(Y, A_not, B_not);
endmodule

# 4. Testbench

module tb;

    reg A, B;
    wire NOT_Y;
    wire AND_Y;
    wire OR_Y;

    not_using_nand NOT1(.A(A), .Y(NOT_Y));
    and_using_nand AND1(.A(A), .B(B), .Y(AND_Y));
    or_using_nand OR1(.A(A), .B(B), .Y(OR_Y));

    initial begin

        $monitor("Time=%0t A=%b B=%b NOT=%b AND=%b OR=%b",
                 $time, A, B, NOT_Y, AND_Y, OR_Y);

        A = 0; B = 0;
        #10;

        A = 0; B = 1;
        #10;

        A = 1; B = 0;
        #10;

        A = 1; B = 1;
        #10;

        $finish;
    end

endmodule

# 5. Simulation Output

Expected simulation output:

Time=0  A=0  B=0  NOT=1  AND=0  OR=0
Time=10 A=0  B=1  NOT=1  AND=0  OR=1
Time=20 A=1  B=0  NOT=0  AND=0  OR=1
Time=30 A=1  B=1  NOT=0  AND=1  OR=1

# 6. Truth Table

| A | B | NOT A | AND | OR |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 0 | 1 |
| 1 | 1 | 0 | 1 | 1 |

# 7. Result

The NOT, AND, and OR gates were successfully implemented using only NAND gates. The simulation outputs match the expected truth tables and Boolean expressions.